UART/FIFO Implementation Description

Overview

This project implements a UART (Universal Asynchronous Receiver/Transmitter) in VHDL for FPGA boards like ZedBoard. It supports 8N1 format (8 data bits, no parity, 1 stop bit) at 115200 baud. The system includes TX/RX cores, FIFOs for buffering, baud rate generator, and a loopback for testing. Clock is 100 MHz. It echoes received data back via TX.

Key Components

- baud_gen.vhd: Generates 16x baud ticks (tick_16x) from system clock. Divisor = CLK_FREQ / (BAUD * 16).

- uart_rx.vhd: Receives serial data on RxD. Uses oversampling to detect start, data, stop bits. Outputs byte on data_o with strobe data_stb_o. Handles frame errors.

- uart_tx.vhd: Transmits byte from data_i. Sends start bit, 8 data bits (LSB first), stop bit. Uses FSM: IDLE, START, DATA, STOP.

- your_fifo.vhd: FWFT FIFO (First Word Fall Through) for buffering. Generic width=8, depth=16. Circular buffer with pointers.

- uart.vhd: Top UART module. Integrates RX/TX cores with FIFOs. User interface: Tx_Data/Tx_Write/Tx_Ready, Rx_Data/Rx_Read/Rx_Valid.

- uart_system_top.vhd: System top. Adds external loopback FIFO. Glue logic moves data: RX -> loopback -> TX.

- uart_tb.vhd: Testbench. Simulates sending test bytes (0x55, 0xA5, etc.) on RxD, monitors TxD for echo.

- Constraints.xdc: Pin constraints for RxD, TxD, rst. Clock period 10 ns.

How It Works

- Baud generator pulses tick_16x every 1/(baud*16) seconds.

- RX: Detects falling edge on RxD (start), samples mid-bit with oversampling, shifts in 8 bits, checks stop bit. Pushes to RX FIFO.

- TX: Pops from TX FIFO, shifts out bits with start/stop. Idle high.

- FIFOs: Buffer data to handle bursts. FWFT shows first word without read.

- Loopback: RX data pushed to external FIFO, then to TX when ready.

- Reset: Synchronous active-high rst clears all.

Testing

- Simulation: uart_tb sends bytes, checks echo on TxD. Reports timings and errors.

- Hardware: Deploy on FPGA with Pmod UART. Send data via serial terminal; it echoes back.

This setup is simple, reliable for serial comms. No parity used; frame errors detected but not output.